Subject: Re: Conv Engine Clock Frequency Estimate — SG13G2 PDK

Dear Professor,

Thank you for confirming the non-pipelined approach for the conv_engine — that simplifies the implementation significantly.

Regarding expected clock frequency with the IHP SG13G2 130nm PDK:

Based on the PDK characteristics and the critical path in our design, a realistic target is 100–200 MHz for a non-pipelined conv_engine. The reasoning is as follows:

- The SG13G2 130nm process typically supports standard-cell logic at 200–400 MHz for simple datapaths, but our critical path includes a multi-operand addition (4-lane MAC accumulation into a 32-bit accumulator) plus SRAM read latency, which will limit achievable frequency.

- For a non-pipelined design where the full MAC-accumulate-write sequence must complete in a single cycle, 100 MHz (10 ns cycle time) is a conservative and safe target that should close timing comfortably.

- 200 MHz may be achievable depending on the synthesized critical path, but would require careful constraint setup and possibly some manual optimization.

I plan to target 100 MHz as the primary constraint for synthesis, and report the achievable frequency from the post-synthesis timing report as the actual result.

Please let me know if you would like me to target a different frequency or run timing analysis at multiple corners.

Best regards,

[Your Name]